# Automatic Voxel-Grid Size Estimation
for Intersecting Robot Workspaces

Prior approximation from raw XYZ workspace CSV files — without repeated voxelization

Technical note with formulas and TypeScript implementation

Recommended approach Estimate the native point spacing of both workspace point clouds, estimate their possible overlap using nearest-neighbor proximity (KD-tree), and analytically derive one voxel size that should yield a desired number of occupied intersection voxels. The actual voxelization can then be executed once.

# 1. Problem Statement

Two robot working spaces are stored as CSV point clouds. Every row represents a reachable Cartesian point p = (x, y, z). A voxel grid is later used to identify the spatial overlap between the two workspaces, i.e. candidate handover regions. A fixed voxel size is undesirable: a small physical overlap may produce too few common voxels with a coarse grid, whereas a large overlap may generate an unnecessarily large number of intersection voxels with a very fine grid.

The requirement considered here is stronger than adaptive multi-resolution voxelization: estimate a suitable voxel size before constructing the voxel grid, using only the raw XYZ coordinates.

# 2. Why Multiple Voxelizations Are Not Required

A convergence-based strategy (voxelize with s, s/2, s/4, … until the overlap stabilizes) is valid, but it necessarily performs several voxelizations. For a prior estimator, information already contained in the point clouds can be used instead:

local sample spacing tells us the useful spatial resolution of the data;

the geometric bounding boxes provide a very cheap first overlap estimate;

nearest-neighbor distances between the two point clouds provide a much better estimate of the true overlap without constructing voxels;

the desired number of intersection voxels can then be converted analytically into a voxel edge length.

# 3. Step 1 — Estimate Native Point-Cloud Spacing

For each point in workspace P, compute the distance to its nearest different point. For point pᵢ:

dᵢ = minⱼ≠ᵢ ‖pᵢ − pⱼ‖₂

Use a robust statistic because occasional duplicate points, singular configurations, or locally dense regions can distort a mean. A median is a simple default:

d_R1 = median({dᵢ | pᵢ ∈ P₁}),     d_R2 = median({dᵢ | pᵢ ∈ P₂})

d_eff = max(d_R1, d_R2)

d_eff is a practical lower useful scale: creating voxels far smaller than the more sparsely sampled workspace normally creates resolution that is not supported by the source data.

If Cartesian point density varies strongly, for example because the workspace was generated by regular sampling in joint space rather than Cartesian space, use a higher robust percentile instead of the median:

d_eff = max(Q₀.₇₅(D_R1), Q₀.₇₅(D_R2))

Here Q₀.₇₅ denotes the 75th percentile of nearest-neighbor distances.

# 4. Step 2 — Very Cheap Bounding-Box Pre-Estimate

For each workspace, compute its axis-aligned bounding box. For robot 1:

B₁ = [x₁,min, x₁,max] × [y₁,min, y₁,max] × [z₁,min, z₁,max]

The overlap length along x is:

Lₓ = max(0, min(x₁,max, x₂,max) − max(x₁,min, x₂,min))

Analogously obtain Lᵧ and L_z. The bounding-box overlap volume is:

V_bbox = Lₓ · Lᵧ · L_z

For a desired number N_target of intersection voxels, a first voxel-size estimate follows from V ≈ N·s³:

s_bbox ≈ ∛(V_bbox / N_target)

s₀ = max(d_eff, s_bbox)

This is extremely cheap, but it can substantially overestimate overlap for curved, shell-like, disconnected, or highly non-convex robot workspaces. Therefore it is best treated as a fallback or initial scale estimate, not as the preferred final prior estimator.

# 5. Step 3 — KD-Tree Proximity Estimate of the Real Overlap

A better prior estimate uses the raw point coordinates directly. Build a KD-tree for P₂. For each point pᵢ in P₁, query the distance to its nearest point in P₂:

δᵢ¹→² = min_{q ∈ P₂} ‖pᵢ − q‖₂

Treat pᵢ as belonging to a candidate common region when its nearest point in the other workspace is within a proximity radius r. A simple radius is tied to the sampling scale:

r = κ · d_eff,     with κ ≈ 1 … 1.5

Count the candidate points:

N_close = |{pᵢ ∈ P₁ : δᵢ¹→² ≤ r}|

Under approximately uniform volumetric sampling, one raw point represents a characteristic spatial volume proportional to d_eff³. Thus:

V̂_intersection ≈ N_close · d_eff³

Demanding approximately N_target occupied intersection voxels yields:

N_target ≈ V̂_intersection / s³

s_est ≈ d_eff · ∛(N_close / N_target)

Finally impose the sampling-resolution lower bound:

s_voxel = max(d_eff, s_est)

# 6. Optional Improvements to the Prior Estimate

Bidirectional proximity: repeat the query P₂ → P₁. This reduces bias when one CSV is much denser than the other.

Subsampling: estimate spacing and overlap using a random subset (e.g. 5–20% of points), then scale the close-point count by the sampling fraction. This can make the pre-estimation very fast.

Robust spacing: use the 75th percentile of nearest-neighbor spacing when Cartesian sampling is non-uniform.

Clamp voxel size: define engineering limits s_min and s_max from robot accuracy, object size, handover tolerance, and memory constraints.

Minimum physical resolution: if the robot positioning uncertainty is ±ε, a voxel size much smaller than ε may be misleading even if the point cloud is dense.

A useful clamped result is therefore:

s_final = clamp(max(d_eff, s_est), s_min, s_max)

# 7. Shared Voxel-Grid Origin Is Mandatory

When the final voxelization is executed, both workspaces must use the same grid origin. Otherwise equal voxel indices do not refer to the same physical volume. Define a common origin component-wise from both point clouds, for example:

o = min(P₁ ∪ P₂)   (component-wise)

Map a point p into voxel coordinates using:

v(p) = ⌊(p − o) / s_final⌋

The floor operation is applied independently to x, y, and z.

# 8. Recommended Algorithm

1. Load both CSV files into arrays of XYZ points.

2. Estimate nearest-neighbor spacing d_R1 and d_R2 from each point cloud (preferably from a random sample).

3. Compute d_eff as the maximum median or maximum 75th percentile spacing.

4. Build a KD-tree for the second workspace and estimate N_close using radius r = κ·d_eff.

5. Optionally repeat in the reverse direction and combine the two estimates.

6. Calculate s_est = d_eff·∛(N_close/N_target).

7. Apply sampling, physical-accuracy, and user-defined minimum/maximum bounds.

8. Perform the actual voxelization once using s_final and one shared grid origin.

9. If the one-shot result is pathological (e.g. zero intersection despite known overlap), fall back to one corrective re-estimation rather than a full resolution sweep.

# 9. TypeScript Implementation

The following example uses the npm package kd-tree-javascript for nearest-neighbor queries. CSV parsing is kept dependency-light. Distances and coordinates must use the same units (e.g. all millimetres or all metres).

// npm install kd-tree-javascript
// npm install --save-dev @types/node

import { readFileSync } from "node:fs";
import { kdTree } from "kd-tree-javascript";

export interface Point3D {
  x: number;
  y: number;
  z: number;
}

export interface VoxelEstimateOptions {
  targetIntersectionVoxels: number;
  proximityFactor?: number;      // kappa, default 1.25
  sampleSize?: number;           // max points used per estimate
  spacingPercentile?: number;    // 0.50 = median, 0.75 for non-uniform density
  minVoxelSize?: number;
  maxVoxelSize?: number;
  bidirectional?: boolean;
}

export interface VoxelEstimateResult {
  spacingRobot1: number;
  spacingRobot2: number;
  effectiveSpacing: number;
  proximityRadius: number;
  estimatedClosePoints: number;
  estimatedVoxelSize: number;
  finalVoxelSize: number;
}

## 9.1 CSV loading and basic utilities

export function loadXYZCsv(
  path: string,
  columns = { x: 0, y: 1, z: 2 },
  hasHeader = true,
): Point3D[] {
  const lines = readFileSync(path, "utf8")
    .split(/\r?\n/)
    .filter(line => line.trim().length > 0);

  const dataLines = hasHeader ? lines.slice(1) : lines;
  const points: Point3D[] = [];

  for (const line of dataLines) {
    const cells = line.split(/[;,\t]/).map(v => v.trim());
    const p = {
      x: Number(cells[columns.x]),
      y: Number(cells[columns.y]),
      z: Number(cells[columns.z]),
    };

    if (Number.isFinite(p.x) && Number.isFinite(p.y) && Number.isFinite(p.z)) {
      points.push(p);
    }
  }

  return points;
}

function euclidean(a: Point3D, b: Point3D): number {
  return Math.hypot(a.x - b.x, a.y - b.y, a.z - b.z);
}

function percentile(values: number[], q: number): number {
  if (values.length === 0) throw new Error("Empty value array");
  const sorted = [...values].sort((a, b) => a - b);
  const pos = (sorted.length - 1) * q;
  const lo = Math.floor(pos);
  const hi = Math.ceil(pos);
  if (lo === hi) return sorted[lo];
  const t = pos - lo;
  return sorted[lo] * (1 - t) + sorted[hi] * t;
}

function randomSample<T>(items: T[], maxCount: number): T[] {
  if (items.length <= maxCount) return [...items];
  const result = [...items];
  for (let i = result.length - 1; i > 0; --i) {
    const j = Math.floor(Math.random() * (i + 1));
    [result[i], result[j]] = [result[j], result[i]];
  }
  return result.slice(0, maxCount);
}

## 9.2 Estimate native sampling spacing

function estimateSpacing(
  points: Point3D[],
  sampleSize = 10_000,
  q = 0.50,
): number {
  if (points.length < 2) throw new Error("At least two points are required");

  const tree = new kdTree<Point3D>(points, euclidean, ["x", "y", "z"]);
  const sample = randomSample(points, sampleSize);
  const nearestDistances: number[] = [];

  for (const p of sample) {
    // Ask for two neighbors because the nearest one is normally p itself.
    const neighbors = tree.nearest(p, 2);
    const nonZero = neighbors
      .map(([, distance]) => distance)
      .filter(distance => distance > 1e-12);

    if (nonZero.length > 0) nearestDistances.push(Math.min(...nonZero));
  }

  if (nearestDistances.length === 0) {
    throw new Error("Could not estimate non-zero nearest-neighbor spacing");
  }

  return percentile(nearestDistances, q);
}

## 9.3 Estimate candidate overlap without voxelization

function estimateClosePointCount(
  source: Point3D[],
  target: Point3D[],
  radius: number,
  sampleSize = 20_000,
): number {
  const tree = new kdTree<Point3D>(target, euclidean, ["x", "y", "z"]);
  const sample = randomSample(source, sampleSize);
  let closeInSample = 0;

  for (const p of sample) {
    const nearest = tree.nearest(p, 1, radius);
    if (nearest.length > 0 && nearest[0][1] <= radius) {
      closeInSample++;
    }
  }

  const closeFraction = closeInSample / sample.length;
  return closeFraction * source.length;
}

## 9.4 One-shot voxel-size estimator

export function estimateVoxelSize(
  robot1: Point3D[],
  robot2: Point3D[],
  options: VoxelEstimateOptions,
): VoxelEstimateResult {
  const {
    targetIntersectionVoxels,
    proximityFactor = 1.25,
    sampleSize = 20_000,
    spacingPercentile = 0.50,
    minVoxelSize = 0,
    maxVoxelSize = Number.POSITIVE_INFINITY,
    bidirectional = true,
  } = options;

  if (targetIntersectionVoxels <= 0) {
    throw new Error("targetIntersectionVoxels must be > 0");
  }

  const d1 = estimateSpacing(robot1, sampleSize, spacingPercentile);
  const d2 = estimateSpacing(robot2, sampleSize, spacingPercentile);
  const dEff = Math.max(d1, d2);
  const radius = proximityFactor * dEff;

  const close12 = estimateClosePointCount(robot1, robot2, radius, sampleSize);
  let closeEstimate = close12;

  if (bidirectional) {
    const close21 = estimateClosePointCount(robot2, robot1, radius, sampleSize);

    // Convert each directional result to an overlap fraction and use the
    // conservative smaller fraction. This reduces density-asymmetry bias.
    const f12 = close12 / robot1.length;
    const f21 = close21 / robot2.length;
    const f = Math.min(f12, f21);

    // Express the estimate in units of points at the more sparsely sampled set.
    closeEstimate = f * Math.min(robot1.length, robot2.length);
  }

  // Analytical estimate: s = d_eff * cbrt(N_close / N_target).
  // Do not go below native sample spacing.
  const ratio = Math.max(closeEstimate / targetIntersectionVoxels, 1);
  const sEst = dEff * Math.cbrt(ratio);

  const lowerBound = Math.max(dEff, minVoxelSize);
  const finalSize = Math.min(Math.max(sEst, lowerBound), maxVoxelSize);

  return {
    spacingRobot1: d1,
    spacingRobot2: d2,
    effectiveSpacing: dEff,
    proximityRadius: radius,
    estimatedClosePoints: closeEstimate,
    estimatedVoxelSize: sEst,
    finalVoxelSize: finalSize,
  };
}

## 9.5 Example usage

const workspace1 = loadXYZCsv("robot1.csv");
const workspace2 = loadXYZCsv("robot2.csv");

const estimate = estimateVoxelSize(workspace1, workspace2, {
  targetIntersectionVoxels: 1_000,
  proximityFactor: 1.25,
  sampleSize: 20_000,
  spacingPercentile: 0.75, // robust choice for non-uniform Cartesian density
  minVoxelSize: 0,         // set according to units / robot accuracy if known
  maxVoxelSize: Infinity,
  bidirectional: true,
});

console.log(estimate);
console.log(`Recommended voxel edge length: ${estimate.finalVoxelSize}`);

# 10. Optional Bounding-Box Fallback in TypeScript

interface Bounds3D {
  min: Point3D;
  max: Point3D;
}

function bounds(points: Point3D[]): Bounds3D {
  const xs = points.map(p => p.x);
  const ys = points.map(p => p.y);
  const zs = points.map(p => p.z);
  return {
    min: { x: Math.min(...xs), y: Math.min(...ys), z: Math.min(...zs) },
    max: { x: Math.max(...xs), y: Math.max(...ys), z: Math.max(...zs) },
  };
}

function bboxOverlapVolume(a: Bounds3D, b: Bounds3D): number {
  const lx = Math.max(0, Math.min(a.max.x, b.max.x) - Math.max(a.min.x, b.min.x));
  const ly = Math.max(0, Math.min(a.max.y, b.max.y) - Math.max(a.min.y, b.min.y));
  const lz = Math.max(0, Math.min(a.max.z, b.max.z) - Math.max(a.min.z, b.min.z));
  return lx * ly * lz;
}

function bboxVoxelEstimate(
  p1: Point3D[],
  p2: Point3D[],
  targetVoxels: number,
  effectiveSpacing: number,
): number {
  const volume = bboxOverlapVolume(bounds(p1), bounds(p2));
  if (volume <= 0) return effectiveSpacing;
  return Math.max(effectiveSpacing, Math.cbrt(volume / targetVoxels));
}

# 11. Important Assumptions and Limitations

The analytical N_close·d_eff³ model assumes that CSV points approximately sample a 3-D volume. If the CSV contains only a workspace surface, the cubic-volume model is not appropriate.

If the point density changes substantially across the workspace, a single d_eff is only a global approximation. Local-density estimation (e.g. k-nearest-neighbor radius) is more accurate but more complex.

The proximity factor κ is an engineering tolerance. κ = 1.0 is strict; 1.25–1.5 is more tolerant of slightly shifted sample lattices.

N_target should reflect the downstream task. If the intersection points are used to optimize a handover point, hundreds to a few thousand occupied voxels are often more useful than an arbitrary fixed spatial resolution.

A guaranteed “optimal” voxel size cannot be derived from point coordinates alone without defining what adequacy means. The estimator therefore needs a target such as desired intersection voxel count, maximum geometric error, or handover-location tolerance.

If the two workspaces are expressed in different robot-local coordinate systems, transform both into the same world/reference frame before all distance, overlap, and voxel computations.

# 12. Recommended Default Configuration

| Parameter | Suggested default | Reason |
| --- | --- | --- |
| spacingPercentile | 0.50 (median) or 0.75 | 0.75 is safer for variable Cartesian density. |
| proximityFactor κ | 1.25 | Tolerates small sampling-lattice offsets without being excessively permissive. |
| sampleSize | 10,000–20,000 points | Usually enough for stable robust statistics while keeping KD-tree queries cheap. |
| N_target | ~500–2,000 | Reasonable starting range for a handover candidate region; tune to downstream optimizer. |
| bidirectional | true | Reduces bias when point clouds differ in density. |
| s_min | ≥ robot/measurement accuracy | Avoids claiming unsupported physical precision. |

# 13. Final Recommendation

For the described robot-workspace problem, the preferred prior approximation is: estimate native point spacing → estimate cross-workspace proximity with a KD-tree → analytically convert the estimated overlap population into one voxel size → voxelize once. This satisfies the goal of avoiding a trial-and-error sweep over multiple voxel sizes while still adapting automatically to small and large workspace intersections.

s_final = clamp(max(d_eff, d_eff · ∛(N_close / N_target)), s_min, s_max)

For non-uniform workspace sampling, use the 75th percentile of nearest-neighbor spacing and bidirectional proximity estimates. If later experiments show that the handover optimum is sensitive to voxel resolution, the target should be reformulated in terms of maximum acceptable handover-position error rather than only the number of intersection voxels.