# Automatic Voxel-Grid Size Approximation for Intersecting Robot Workspaces

Method, Formula Derivation, Implementation, and Scientific Context

WoMAT / Master’s Thesis Technical Note

17 August 2026

# Automatic Voxel-Grid Size Approximation for Intersecting Robot Workspaces

Method, Formula Derivation, Implementation, and Scientific Context

WoMAT / Master’s Thesis Technical Note
17 August 2026

# Contents

- Abstract

- 1. Problem definition

- 2. Input model and coordinate frames

- 3. Native point-cloud spacing

- 4. Bounding-box overlap gate

- 5. Proximity approximation

- 6. Deriving voxel edge length

- 7. One shared voxel grid

- 8. Complete algorithm

- 9. Current Robot1/Robot2 result

- 10. Why this is a prior

- 11. Parameter interpretation

- 12. Computational considerations

- 13. Recommended thesis evaluation

- 14. Scientific context

- 15. Implementation and reproduction

- 16. Suggested future improvements

- References

# Abstract

This technical note documents the one-shot voxel-grid size approximation used for the interaction space between two robot workspaces in WoMAT. Each workspace is available as a Cartesian point cloud of reachable tool-centre-point (TCP) positions. The objective is to choose a voxel edge length before voxelising, then voxelise each workspace exactly once on one shared grid and intersect the occupied voxel indices.

The method combines four ideas:

- nearest-neighbour distances estimate the useful spatial resolution supported by each point cloud;

- a KD-tree estimates how many raw samples lie near the other robot’s workspace without constructing a voxel grid;

- the volume identity V≈Ns3 converts a desired interaction-voxel count into a voxel edge length; and

- an explicit common origin ensures that equal voxel indices denote the same physical cells.

The final cube-root estimator is a project-specific engineering derivation. It is not presented as a formula copied from one scientific article. Its components are consistent with established nearest-neighbour density estimation, multidimensional search, point-cloud processing, and voxelized robot workspace representations (Loftsgaarden and Quesenberry 1965; Bentley 1975; Rusu and Cousins 2011; Zacharias, Borst, and Hirzinger 2007).

# 1. Problem definition

Let the reachable TCP samples of two robots be

P1={p1,i}i=1n1,  P2={p2,j}j=1n2,

where every point is expressed in metres and p=x,y,zT∈R3. The intended result is an interaction space

Is=VsP1∩VsP2,

where VsP is the set of occupied voxel indices generated from point set P using edge length s.

The design target is not “the mathematically optimal voxel size,” because that quantity is undefined until an optimization criterion is chosen. Instead, the configuration specifies a desired number of occupied interaction voxels, Ntarget. This is a computational and task-level resolution target: it controls the approximate number of handover candidates and therefore the cost and granularity of subsequent selection.

Voxelized workspace and capability maps are established representations for robot-arm reachability (Zacharias, Borst, and Hirzinger 2007; Anderson-Sprecher and Simmons 2012). The special problem addressed here is choosing the resolution automatically from already sampled workspaces.

# 2. Input model and coordinate frames

## 2.1 Reachable samples

The WoMAT workspace CSV format contains

X,Y,Z,Collision_state

Only rows with Collision_state = 1 are used. Non-finite rows are rejected and exact duplicate XYZ points are removed before spacing estimation. The method assumes that these samples approximate a three-dimensional reachable volume. If the CSV contains only a two-dimensional boundary surface, the cubic volume model in Section 6 is inappropriate.

## 2.2 Transforming each workspace into the world frame

The workspace files store robot-local coordinates. For robot r, let its base translation be

tr=tx,r,ty,r,tz,rT

and its yaw rotation be θr. The world point is

pW=tr+RzθrpL,

with

Rzθ=cosθ−sinθ0sinθcosθ0001.

All distance, bounding-box, proximity, origin, and voxel calculations are performed after this transformation. Mixing local frames would produce a geometrically meaningless intersection.

# 3. Step 1 — native point-cloud spacing

For every distinct point pi in workspace Pr, define the distance to the nearest different point:

dr,i=minj≠ipr,i−pr,j2.

The project uses a robust percentile rather than the arithmetic mean:

dr=Qq{dr,i},

where Qq denotes quantile q. The default is q=0.75. A median (q=0.50) is suitable for approximately uniform Cartesian sampling; the 75th percentile is more conservative when Cartesian density varies because the original samples were generated in joint space.

The effective supported spacing is

deff=maxd1,d2.

Taking the maximum prevents the denser point cloud from claiming a resolution that the sparser cloud does not support. Nearest-neighbour distances are a classical basis for multivariate density estimation (Loftsgaarden and Quesenberry 1965). Local sampling density is also central to point-cloud approximation and simplification (Pauly, Gross, and Kobbelt 2002). Here, however, deff is used as an engineering lower resolution bound, not as an unbiased statistical estimator.

For large clouds, at most m points are selected with a deterministic seeded shuffle. Defaults are m=20,000 and seed 42. Deterministic sampling makes the result reproducible.

# 4. Step 2 — bounding-box overlap gate

For workspace Pr, define the axis-aligned bounding box

Br=xr,min,xr,max×yr,min,yr,max×zr,min,zr,max.

The overlap length on axis a∈{x,y,z} is

La=max0,mina1,max,a2,max−maxa1,min,a2,min.

Therefore,

Vbbox=LxLyLz.

If Vbbox=0, the implementation returns no_geometric_overlap without building the proximity estimator. If it is positive, the value is only a coarse upper proxy. Curved, hollow, disconnected, or non-convex workspaces can occupy far less volume than their boxes.

# 5. Step 3 — proximity approximation without voxelisation

A three-dimensional KD-tree is built for the target point cloud. KD-trees were introduced for efficient multidimensional associative search (Bentley 1975) and are used here for nearest-neighbour and fixed-radius queries.

For every sampled point in direction P1→P2, define

δi1→2=minq∈P2p1,i−q2.

The proximity radius is coupled to native resolution:

ρ=κdeff,

where κ=1.25 by default. A point is “close” when δi1→2≤ρ. If c12 of m1 sampled points are close, the full-cloud count estimate is

C12=c12m1n1.

The calculation is repeated in reverse to reduce density-asymmetry bias:

C21=c21m2n2.

WoMAT combines the directional overlap fractions conservatively:

Nclose=minC12n1,C21n2minn1,n2.

This symmetrization is a project-specific choice. It suppresses a large result when only the denser cloud reports a high close-point fraction.

# 6. Step 4 — deriving voxel edge length

## 6.1 Characteristic sample volume

If the point clouds approximately sample a 3-D volume, a characteristic sample spacing deff corresponds dimensionally to a characteristic volume

vsample∝deff3.

WoMAT chooses the proportionality constant as one and approximates interaction volume by

Vint=Nclosedeff3.

This is deliberately a prior approximation. It is not an exact union-volume estimator, and no claim is made that every sample owns a disjoint cube of side deff.

## 6.2 Target voxel count

A cubic voxel of edge length s has exact volume

Vvoxel=s3.

Ignoring boundary and occupancy effects, a volume V contains approximately

N≈Vs3

voxels. Solving for s gives

s≈3VN.

Substituting Vint and the configured Ntarget yields

sest=deff3NcloseNtarget.

The implementation does not refine below native sampling resolution. Equivalently,

sest=deff3maxNcloseNtarget,1.

The cube root is not an empirical fitted exponent: it follows from using cubic cells in three spatial dimensions. In general dimension D, the corresponding scaling would be N−1/D.

## 6.3 Bounding-box fallback

If the bounding boxes overlap but the sampled proximity count is zero, one fallback estimate is used:

sbbox=maxdeff,3VbboxNtarget.

The algorithm then performs the normal single voxelisation. It does not run a resolution sweep.

## 6.4 Engineering bounds

The final value is

sfinal=minsmax,maxsest,deff,smin.

smin can represent robot accuracy, calibration uncertainty, or the minimum meaningful handover tolerance. smax limits geometric coarseness. The configuration is rejected if smax<maxdeff,smin.

# 7. Step 5 — one shared voxel grid

The shared origin is the component-wise minimum over both world-space clouds:

o=minp∈P1∪P2pxminp∈P1∪P2pyminp∈P1∪P2pz.

Every point is mapped to an integer voxel index:

vp=p−osfinal,

where subtraction, division, and floor are component-wise. The occupied sets are

V1={vp:p∈P1},  V2={vp:p∈P2}.

The discrete interaction space is the set intersection

I=V1∩V2.

For an interaction index k=kx,ky,kz, its world-space centre is

ck=o+sfinalk+121.

The common origin is mandatory. Using ⌊p/s⌋ independently may happen to align two grids only when both silently share world zero; it does not express the invariant and is fragile under coordinate changes.

# 8. Complete algorithm

INPUT:
  reachable point clouds P1_local, P2_local
  base transforms T1, T2
  target count N_target
  percentile q, proximity factor kappa, sample limit m
  bounds s_min, s_max, deterministic seed

1. Filter valid reachable rows and remove exact duplicates.
2. Transform both point clouds into the world frame.
3. Compute AABBs; stop if their overlap volume is zero.
4. Estimate d1 and d2 from sampled nearest-neighbour distances.
5. Set d_eff = max(d1, d2) and rho = kappa * d_eff.
6. Estimate close-point counts in both directions with KD-trees.
7. Conservatively combine the two close fractions.
8. Compute s_est from d_eff * cbrt(N_close / N_target), with a
   lower bound of d_eff; use the bounding-box fallback only if necessary.
9. Clamp against engineering limits and validate the result.
10. Compute one shared component-wise origin.
11. Voxelise P1 once and P2 once using the same size and origin.
12. Intersect their occupied integer-key sets.
13. Store voxel centres and estimator diagnostics in Robot 1's private
    interaction-space controller; publish summary beliefs to Robot 1.

# 9. Current Robot1/Robot2 result

For the workspace files present on 17 August 2026, the deterministic default configuration produced:

| Quantity | Value |
| --- | --- |
| Reachable Robot 1 samples | 51,663 |
| Reachable Robot 2 samples | 62,924 |
| Robot 1 spacing, d1 | 0.0356716 m |
| Robot 2 spacing, d2 | 0.0138473 m |
| Effective spacing, deff | 0.0356716 m |
| Proximity radius, ρ | 0.0445895 m |
| Estimated close points | 1,358.7369 |
| Target interaction voxels | 1,000 |
| Estimated/final voxel size | 0.0395095 m |
| Actual occupied interaction voxels | 967 |
| Estimator mode | KD-tree proximity |

The shared origin was

o=−1.2995830,−1.3036553,−1.1727232T m.

Obtaining 967 instead of exactly 1,000 is expected. The formula estimates continuous volume from discrete samples; boundary placement, duplicate occupancy, nonuniform density, and workspace geometry determine the final integer count.

# 10. Why this is a prior, not an optimum

The estimator answers: “Which voxel size is likely to produce roughly the desired candidate count without repeatedly voxelising?” It does not solve a universal optimization problem. Important distinctions are:

- Task target: Ntarget is chosen for downstream usefulness, not inferred from geometry alone.

- Resolution support: deff limits false precision, but robot calibration uncertainty may require a larger lower bound.

- Occupancy approximation: a sampled point causes its whole voxel to count as occupied; this can overestimate the continuous intersection near boundaries.

- Position only: the current grid represents TCP position in R3, not orientation in SO3 or complete pose in SE3. Capability maps often discretize orientations as well (Zacharias, Borst, and Hirzinger 2007).

- Reachability, not path feasibility: intersection means both workspaces contain samples in the same cell. Runtime collision checking and motion planning remain necessary.

- Sampling bias: uniform joint-space sampling is generally nonuniform in Cartesian space. A single global percentile cannot fully correct this.

# 11. Parameter interpretation

| Parameter | Default | Effect of increasing it |
| --- | --- | --- |
| Ntarget | 1,000 | Smaller voxels, more candidates and memory |
| q | 0.75 | More conservative/larger native spacing |
| κ | 1.25 | More points classified as near the other workspace |
| sample size m | 20,000 | Lower sampling variance, higher pre-estimation time |
| smin | 0 m | Enforces physical/accuracy lower resolution |
| smax | 0.2 m | Caps excessive coarseness |
| bidirectional | true | Reduces asymmetric-density bias |

Sensitivity follows directly from the cube root. For example, multiplying the estimated overlap count by eight multiplies the estimated voxel edge by two. Likewise, multiplying Ntarget by eight divides the edge length by two, before lower-bound clamping.

# 12. Computational considerations

With n points and a balanced KD-tree, nearest and radius queries are efficient on typical low-dimensional data, although worst-case query time remains linear. The present JavaScript implementation recursively sorts point subsets, giving approximately Onlog2n construction in this implementation rather than an optimized linear-time median build. Estimation queries are performed only for the deterministic sample of at most m points.

After estimating s, voxelisation is linear in the number of raw points:

Tvoxel=On1+n2.

Hash-set intersection is linear in the smaller occupied set on average. Memory is proportional to the number of points during estimation and to the number of occupied cells during voxelisation. More sophisticated octree representations, such as OctoMap, are valuable when probabilistic occupancy, multi-resolution queries, or large environments are required (Hornung et al. 2013). WoMAT deliberately uses a single-resolution hash grid because it needs a simple, reproducible set intersection.

# 13. Recommended thesis evaluation

The method should be evaluated empirically rather than justified only by the current successful example.

## 13.1 Baselines

Compare against:

- the former fixed size, 0.05 m;

- the bounding-box-only estimate;

- the proposed bidirectional proximity estimate; and

- an offline multi-resolution sweep used only as an evaluation reference.

## 13.2 Metrics

- absolute and relative error from Ntarget;

- estimator time and one-shot voxelisation time;

- occupied-cell memory;

- Jaccard similarity of interaction cells against a fine-grid reference;

- stability across random workspace samples and estimator seeds;

- stability under small robot-base calibration perturbations;

- number of candidates passing both robots’ final workspace checks;

- end-to-end handover planning success and motion success.

For two voxel sets A and B, Jaccard similarity is

JA,B=A∩BA∪B.

When resolutions differ, compare physical cell volumes or resample both results onto a common evaluation grid before computing J.

## 13.3 Sensitivity study

Recommended factors are

q∈{0.50,0.75,0.90},  κ∈{1.0,1.25,1.5},

and

Ntarget∈{250,500,1000,2000,4000}.

Report not only the mean but also dispersion over repeated point-cloud generation runs. This will expose whether the apparent precision of a single voxel size is justified by the sampling process.

# 14. Scientific context and boundaries of support

| Component in WoMAT | Scientific support | What is specific to WoMAT |
| --- | --- | --- |
| Voxelized robot reachability | Robot workspace/capability maps discretize reachable regions (Zacharias, Borst, and Hirzinger 2007; Anderson-Sprecher and Simmons 2012) | Intersection is used to form a handover interaction space |
| Nearest-neighbour spacing | Nearest-neighbour radii support multivariate density estimation (Loftsgaarden and Quesenberry 1965) | Maximum robust percentile becomes a resolution floor |
| Efficient proximity queries | KD-trees support multidimensional search (Bentley 1975) | Deterministic subsampling and bidirectional conservative fraction |
| Point-cloud processing | PCL establishes practical 3-D point-cloud processing infrastructure (Rusu and Cousins 2011) | Dependency-free JavaScript implementation |
| Cube-root scaling | Exact cubic cell volume s3 | V=Nclosedeff3 and target-count inversion |
| Occupancy structures | OctoMap demonstrates efficient 3-D voxel/octree mapping (Hornung et al. 2013) | Single-resolution hash sets are chosen for exact set intersection |

Thus, a defensible thesis statement is:

The proposed voxel-size approximation is an engineering estimator derived from dimensional voxel-volume scaling and a nearest-neighbour proxy for overlap volume. Its constituent methods are established, while the combined estimator and its parameterization require empirical validation for the studied robot workspaces.

# 15. Implementation mapping and reproduction

The estimator and Robot 1-owned controller are implemented in:

src/generator/interactionSpace.js

Agent belief creation and local point selection are wired in:

src/generator/agentThingServer.js
src/agt/handoverPlans.asl

The shared lower runtime stores only physical session identity and cube state:

src/generator/lowerThingServer.js

The configuration is stored in:

generated/robots/Robot1_Robot2.shared.json

Reproduce the current calculation without CoppeliaSim:

npm run interaction:estimate

Run the automated tests:

npm test

Refresh the optional green CoppeliaSim visualization after estimation:

npm run robot:pipeline -- \
  --manifests generated/robots/Robot1/manifest.json,generated/robots/Robot2/manifest.json \
  --update-shared-space-visual

# 16. Suggested future improvements

- Estimate local density with k-nearest-neighbour radii rather than one global spacing statistic.

- Model positional uncertainty by dilating occupied cells or using probabilistic occupancy.

- Extend interaction beliefs from position to orientation-aware reachability.

- Validate or learn κ and Ntarget from handover outcomes.

- Record content hashes of workspace CSV inputs in estimator metadata.

- Quantify boundary aliasing by repeating only the evaluation with shifted grid origins; keep one canonical origin in production.

- Compare conservative, optimistic, and probability-weighted definitions of shared occupancy.

# References

Anderson-Sprecher, Peter, and Reid Simmons. 2012. “Voxel-Based Motion Bounding and Workspace Estimation for Robotic Manipulators.” In 2012 IEEE International Conference on Robotics and Automation, 2141–46. https://doi.org/10.1109/ICRA.2012.6225256.

Bentley, Jon Louis. 1975. “Multidimensional Binary Search Trees Used for Associative Searching.” Communications of the ACM 18 (9): 509–17. https://doi.org/10.1145/361002.361007.

Hornung, Armin, Kai M. Wurm, Maren Bennewitz, Cyrill Stachniss, and Wolfram Burgard. 2013. “OctoMap: An Efficient Probabilistic 3D Mapping Framework Based on Octrees.” Autonomous Robots 34 (3): 189–206. https://doi.org/10.1007/s10514-012-9321-0.

Loftsgaarden, D. O., and C. P. Quesenberry. 1965. “A Nonparametric Estimate of a Multivariate Density Function.” The Annals of Mathematical Statistics 36 (3): 1049–51. https://doi.org/10.1214/aoms/1177700079.

Pauly, Mark, Markus Gross, and Leif P. Kobbelt. 2002. “Efficient Simplification of Point-Sampled Surfaces.” In Proceedings Visualization 2002, 163–70. https://doi.org/10.1109/VISUAL.2002.1183771.

Rusu, Radu Bogdan, and Steve Cousins. 2011. “3D Is Here: Point Cloud Library (PCL).” In 2011 IEEE International Conference on Robotics and Automation. https://doi.org/10.1109/ICRA.2011.5980567.

Zacharias, Franziska, Christoph Borst, and Gerd Hirzinger. 2007. “Capturing Robot Workspace Structure: Representing Robot Capabilities.” In 2007 IEEE/RSJ International Conference on Intelligent Robots and Systems, 3229–36. https://doi.org/10.1109/IROS.2007.4399105.